# Experimental Protocol (by Wen)

## Determination of Log P Using the Shake-Flask Method and HPLC Analysis

## 1. Objective

To experimentally determine the partition coefficient (Log P) of a target compound in an n-octanol/water biphasic system. Log P is a key physicochemical parameter indicative of the compound’s lipophilicity and potential membrane permeability.

## 2. Principle

The Log P value represents the logarithm of the ratio of the equilibrium concentrations of a compound in the n-octanol and water phases:

log P = log₁₀(C_octanol / C_water)

After allowing the compound to partition between mutually saturated n-octanol and water, the concentration in each phase is quantified via high-performance liquid chromatography (HPLC).

## 3. Materials and Reagents

| Item | Specification |
| --- | --- |
| n-Octanol | HPLC grade |
| Deionized water or phosphate buffer (pH 7.4) | Laboratory-prepared |
| Target compound (analytical standard) | ≥ 98% purity |
| HPLC system with UV detector | Standard configuration |
| HPLC column | C18 reverse-phase (150 mm × 4.6 mm, 5 μm) |
| Mobile phase | Water/Acetonitrile or Water/Methanol |
| Syringe filters (0.22 μm) | PTFE or nylon |
| Micropipettes and tips | Appropriate volumes |
| Sealed vials | 10–20 mL volume |
| Centrifuge | Low-speed or microcentrifuge |

## 4. Procedure

### Step 1: Solvent Pre-Saturation

1. Mix equal volumes (e.g., 50 mL each) of n-octanol and water in a bottle. (The volume ratio between n-octanol and water is not fixed and should be adjusted based on the physicochemical properties of the compound under investigation.)
2. Shake vigorously or stir at room temperature for 12–24 hours. (For initial determinations, it is advisable to perform time-course monitoring to ensure that partitioning equilibrium has been reached in both the aqueous and organic phases.)
3. Allow to separate into two layers.
4. Collect:
   - Water-saturated n-octanol
   - n-Octanol-saturated water

### Step 2: Partitioning Experiment

1. In a clean vial, add equal volumes of pre-saturated n-octanol and water (e.g., 5 mL each).
2. Add a known amount of the target compound (from a stock solution) to achieve a concentration within the linear detection range of the HPLC.
3. Seal and gently mix using a vortex or rotator for 1–2 hours at room temperature (25°C).
4. Allow the mixture to settle and separate completely. Use centrifugation if emulsion forms.
5. Carefully collect each phase (octanol = top, water = bottom).
6. Filter each phase using a 0.22 μm syringe filter before HPLC analysis.

### Step 3: HPLC Conditions (Example)

| Parameter | Condition |
| --- | --- |
| Column | C18 reverse-phase |
| Mobile phase A | Water with 0.1% TFA |
| Mobile phase B | Acetonitrile with 0.1% TFA |
| Flow rate | 1.0 mL/min |
| Elution | Gradient (e.g., 5% B → 95% B in 5 min) or isocratic |
| Injection volume | 10–20 µL |
| Detection wavelength | Based on compound UV λmax (e.g., 254 nm) |

### Step 4: Data Analysis

1. Record the HPLC peak area (or concentration using a calibration curve) for each phase.
2. Calculate Log P:

   log P = log₁₀(A_octanol / A_water)

   Where:
   - A_octanol: HPLC peak area or concentration in the octanol phase
   - A_water: HPLC peak area or concentration in the water phase

3. Perform the experiment in triplicate and report the mean ± standard deviation.

## 5. Notes and Precautions

- Ensure the compound remains in its neutral (non-ionized) form throughout the experiment (preferably at pH 7.4).
- Both phases must be mutually saturated before use.
- Use appropriate sample dilution to keep peak areas within the linear response range.
- Avoid compound degradation (e.g., due to light, oxidation, or hydrolysis).
- Do not inject undiluted octanol directly into the HPLC system; filter and dilute as needed.
- Use blank solvent runs between injections to prevent carryover.

## 6. References

1. OECD Guidelines for the Testing of Chemicals. Test No. 107: Partition Coefficient (n-octanol/water): Shake Flask Method.
2. Leo, A., Hansch, C., & Elkins, D. (1971). Partition coefficients and their uses. Chem. Rev., 71, 525–616.
3. Snyder, L. R., & Kirkland, J. J. (1979). Introduction to Modern Liquid Chromatography, Wiley.